TaktOS

User Reference Guide

ARM Cortex-M0/M0+ / M4 / M7 / M33 / M55

Rev 1.1 | May 2026 | I-SYST inc.

Baseline: TaktOS_Dev_260505

| Area | Current guide scope |
| --- | --- |
| Native kernel API | Threads, semaphore, mutex, queue, yield, sleep, handoff, stack guard. |
| Runtime layers | POSIX PSE51, ISO C <threads.h> front end, draft C++ runtime hooks. |
| Validation | KVB kernel validation benchmark plus Thread-Metric benchmark projects. |
| Port scope | ARM Cortex-M production path. RISC-V tree is present as a placeholder/design area and is not a v1.x production port. |

# 1. Quick Start

TaktOS is a static-allocation RTOS kernel. The application provides every thread buffer and every synchronization object. The kernel does not allocate from a heap.

## 1.1 Minimum working example - C

#include "TaktOS.h"
#include "TaktOSThread.h"

#define APP_TICK_HZ      1000u
#define TASK_STACK_BYTES 512u

static uint8_t g_task_mem[TAKTOS_THREAD_MEM_SIZE(TASK_STACK_BYTES)] TAKT_ALIGNED(4);

static void MyTask(void *arg)
{
    (void)arg;

    for (;;)
    {
        /* application work */
        TaktOSThreadSleep(TaktOSCurrentThread(), 100u);  /* 100 ms */
    }
}

int main(void)
{
    TaktOSInit(SystemCoreClock, APP_TICK_HZ, TAKTOS_TICK_CLOCK_PROCESSOR, 0u);
    TaktOSThreadCreate(g_task_mem, sizeof(g_task_mem), MyTask, NULL,
                       TAKTOS_PRIORITY_NORMAL);
    TaktOSStart();   /* never returns */
}

NOTE  Use TAKTOS_THREAD_MEM_SIZE(), not a raw stack byte count. The macro adds the TCB, guard word, architecture frame reserve, guard-region reserve, and alignment slack required by the active port.

## 1.2 Minimum working example - C++

#include "TaktOS.h"
#include "TaktOSThread.h"

static uint8_t g_task_mem[TAKTOS_THREAD_MEM_SIZE(512u)] TAKT_ALIGNED(4);
static TaktOSThread g_task;

static void MyTask(void *arg)
{
    (void)arg;

    for (;;)
    {
        TaktOSThreadSleep(TaktOSCurrentThread(), 100u);  /* 100 ms */
    }
}

int main(void)
{
    TaktOSInit(SystemCoreClock, 1000u, TAKTOS_TICK_CLOCK_PROCESSOR, 0u);
    g_task.Create(g_task_mem, sizeof(g_task_mem), MyTask, nullptr,
                  TAKTOS_PRIORITY_NORMAL);
    TaktOSStart();
}

## 1.3 Build setup

| Item | Required setting |
| --- | --- |
| Language | C17 for C sources; C++23 freestanding for C++ kernel/runtime sources. |
| Compiler flags | -fno-exceptions -fno-rtti -ffunction-sections -fdata-sections. |
| Portable include | include/ |
| ARM include | ARM/include/ and the selected ARM/cm*/ assembly source. |
| Portable sources | src/taktos.cpp, taktos_thread.cpp, taktos_sem.cpp, taktos_mutex.cpp, taktos_queue.cpp. |
| ARM source | ARM/src/TaktKernelCM.cpp plus the matching PendSV_*.S file. |
| POSIX/runtime optional | Add src/posix/*.cpp and/or src/std/*.cpp only when those layers are used. |

# 2. Kernel Initialization and Time Model

## 2.1 TaktOSInit

TaktOSErr_t TaktOSInit(uint32_t KernClockHz, uint32_t TickHz,
                       TaktOSTickClockSrc_t TickClockSrc,
                       uintptr_t HandlerBaseAddr);

| Parameter | Description |
| --- | --- |
| KernClockHz | Input clock of the tick peripheral in Hz. This is not always the CPU clock. Pass the clock actually driving SysTick or the target tick source. |
| TickHz | Configured kernel tick rate. Typical embedded value is 1000 Hz. |
| TickClockSrc | TAKTOS_TICK_CLOCK_PROCESSOR for processor-clock SysTick path; TAKTOS_TICK_CLOCK_REFERENCE for a divided/reference path. Non-SysTick ports may ignore it. |
| HandlerBaseAddr | Architecture-defined handler table base. ARM ports use this when installing runtime handler entries into an application-owned RAM vector table. Pass 0 to use statically linked default handlers. |
| Return | TAKTOS_OK on success. Do not call TaktOSStart() after an initialization error. |

## 2.2 Start and tick-rate query

void TaktOSStart(void);       /* never returns */
uint32_t TaktOSGetTickHz(void);

TaktOSStart() starts the tick source, selects the first runnable thread, and performs the initial stack switch. TaktOSGetTickHz() returns the TickHz value passed to TaktOSInit(), or 0 before successful initialization.

## 2.3 Sleep and timeout units

| API | Unit | Use |
| --- | --- | --- |
| TaktOSThreadSleep(hThread, Ms) | milliseconds | Application-facing delay. Converts milliseconds to ticks with round-up. |
| TaktOSThreadSleepTicks(hThread, Ticks) | kernel ticks | Tick-domain state machines, timeout pass-through, benchmarks, and validation tests. |
| TaktOSSemTake / TaktOSMutexLock / TaktOSQueueSend / TaktOSQueueReceive | kernel ticks | Finite blocking timeouts. Use TAKTOS_WAIT_FOREVER or TAKTOS_NO_WAIT for special cases. |

NOTE  The Rev 1.1 public API split changed TaktOSThreadSleep() to milliseconds. Old code that passed ticks must call TaktOSThreadSleepTicks() instead.

| Constant | Meaning |
| --- | --- |
| TAKTOS_NO_WAIT | 0. Return immediately if the operation cannot complete. |
| TAKTOS_WAIT_FOREVER | 0xFFFFFFFF. Block indefinitely in thread context. |

# 3. Threads

## 3.1 Static thread memory

#define STACK_BYTES 768u
static uint8_t g_worker_mem[TAKTOS_THREAD_MEM_SIZE(STACK_BYTES)] TAKT_ALIGNED(4);

hTaktOSThread_t h = TaktOSThreadCreate(g_worker_mem, sizeof(g_worker_mem),
                                       Worker, NULL, TAKTOS_PRIORITY_NORMAL);

The memory block contains the TCB, guard word, architecture reserve, and usable stack. Stack grows down from the top of the block toward the guard word.

## 3.2 Priority levels

| Constant | Value | Intended use |
| --- | --- | --- |
| TAKTOS_PRIORITY_IDLE | 0 | Reserved for the idle thread. Do not use in application code. |
| TAKTOS_PRIORITY_LOWEST | 1 | Background work. |
| TAKTOS_PRIORITY_LOW | 8 | Low-priority processing. |
| TAKTOS_PRIORITY_NORMAL | 16 | General application tasks. |
| TAKTOS_PRIORITY_HIGH | 24 | Time-sensitive processing. |
| TAKTOS_PRIORITY_HIGHEST | 28 | Near real-time response. |
| TAKTOS_PRIORITY_CRITICAL | 31 | Highest user priority. Use sparingly. |

## 3.3 C thread API

| Function | Description |
| --- | --- |
| TaktOSThreadCreate() | Create a thread using caller-provided memory and make it ready. Returns a handle or NULL. |
| TaktOSThreadSuspend() | Suspend a READY or current thread. Self-suspend from ISR is rejected. |
| TaktOSThreadResume() | Move BLOCKED/SLEEPING/SUSPENDED thread to READY. May request preemption. |
| TaktOSThreadSleep() | Sleep for at least the requested milliseconds, rounded up to kernel ticks. |
| TaktOSThreadSleepTicks() | Sleep for a requested number of kernel ticks. Zero returns immediately. |
| TaktOSThreadDestroy() | Remove the thread from scheduler/wait structures. Handle becomes a tombstone. |
| TaktOSThreadHandOff() | Wake a target thread and block the caller in one scheduler operation. |
| TaktOSThreadYield() | Yield to a peer at the same priority when called in normal Thread mode. |
| TaktOSCurrentThread() | Return the current thread handle. |

## 3.4 C++ wrapper

TaktOSThread myThread;
myThread.Create(mem, sizeof(mem), Entry, nullptr, TAKTOS_PRIORITY_NORMAL);
myThread.Sleep(100u);       /* milliseconds */
myThread.SleepTicks(10u);   /* kernel ticks */
myThread.Suspend();
myThread.Resume();
myThread.Destroy();

## 3.5 Yield semantics

- Normal Thread mode, interrupts enabled, peer at same priority: ready ring rotates and PendSV is requested immediately.

- ISR / Handler mode: yield is deferred. The tick handler consumes the deferred request safely.

- Thread mode with PRIMASK set: yield is deferred so the caller critical section is not opened unexpectedly.

# 4. Semaphores

TaktOSSem_t is a counting semaphore. MaxCount sets the ceiling. MaxCount=1 gives a binary semaphore. Waiters are maintained in priority order.

static TaktOSSem_t g_sem;

TaktOSSemInit(&g_sem, 0u, 1u);      /* binary, starts empty */
TaktOSSemGive(&g_sem, false);       /* ISR-safe signal */
TaktOSSemTake(&g_sem, true, TAKTOS_WAIT_FOREVER);

| Function | Description |
| --- | --- |
| TaktOSSemInit(pSem, Initial, MaxCount) | Initialize a caller-allocated semaphore object. |
| TaktOSSemGive(pSem, false) | Post one token. ISR-safe. Returns TAKTOS_ERR_FULL if already at MaxCount. |
| TaktOSSemTake(pSem, bBlocking, TimeoutTicks) | Acquire one token, optionally blocking for a tick timeout. |

NOTE  Use semaphores for ISR-to-thread notification. Do not use mutexes from interrupt context.

# 5. Mutexes

TaktOS provides non-recursive binary mutexes in two forms: plain mutex and priority-ceiling mutex. Both are caller-allocated TaktOSMutex_t objects.

## 5.1 Plain mutex

static TaktOSMutex_t g_lock;

TaktOSMutexInit(&g_lock);
TaktOSMutexLock(&g_lock, true, TAKTOS_WAIT_FOREVER);
/* protected work */
TaktOSMutexUnlock(&g_lock);

## 5.2 Priority-ceiling mutex (IPCP)

TaktOSMutexInitProtect() creates an Immediate Priority Ceiling Protocol mutex, equivalent in intent to POSIX PTHREAD_PRIO_PROTECT. On lock, the holder is boosted to max(current priority, ceiling). On unlock, the holder priority is restored from the held-ceiling stack.

static TaktOSMutex_t g_pcp;

TaktOSMutexInitProtect(&g_pcp, TAKTOS_PRIORITY_HIGH);
TaktOSMutexLock(&g_pcp, true, TAKTOS_WAIT_FOREVER);
/* bounded critical section */
TaktOSMutexUnlock(&g_pcp);

| Rule | Effect |
| --- | --- |
| Ceiling range | 1..31. Ceiling 0 is rejected. |
| Caller priority | Caller priority must be <= ceiling or Lock returns TAKTOS_ERR_INVALID. |
| Nesting limit | TAKTOS_MAX_HELD_PCP_MUTEXES, default 4, limits simultaneous priority-ceiling mutexes held by one thread. |
| Recursive lock | Not supported. A thread must not lock a mutex it already owns. |
| ISR usage | Mutex lock/unlock are thread-context APIs. Use a semaphore for ISR notification. |

| Function | Description |
| --- | --- |
| TaktOSMutexInit(pMtx) | Initialize an unlocked plain mutex. |
| TaktOSMutexInitProtect(pMtx, Ceiling) | Initialize an unlocked IPCP mutex with the given ceiling. |
| TaktOSMutexLock(pMtx, bBlocking, TimeoutTicks) | Acquire the mutex or return BUSY/TIMEOUT/INVALID. |
| TaktOSMutexUnlock(pMtx) | Release the mutex. Non-owner unlock returns TAKTOS_ERR_INVALID. |

# 6. Message Queues

TaktOSQueue_t is a fixed-size ring buffer. All items in one queue have the same byte size. The backing storage is provided by the application.

typedef struct {
    uint32_t sensor_id;
    uint32_t value;
    uint32_t timestamp;
    uint32_t flags;
} Message_t;

static TaktOSQueue_t g_queue;
static uint8_t g_storage[8u * sizeof(Message_t)] TAKT_ALIGNED(4);

TaktOSQueueInit(&g_queue, g_storage, sizeof(Message_t), 8u);
TaktOSQueueSend(&g_queue, &msg, true, TAKTOS_WAIT_FOREVER);
TaktOSQueueReceive(&g_queue, &msg, true, TAKTOS_WAIT_FOREVER);

| Requirement | Reason |
| --- | --- |
| Storage alignment | pStorage must be 4-byte aligned. |
| Item size | ItemSize must be non-zero and a multiple of 4. |
| Send/receive pointer | pData must be 4-byte aligned. |
| ISR use | Use non-blocking operations from ISR context. Blocking from ISR is rejected. |

| Function | Description |
| --- | --- |
| TaktOSQueueInit(pQue, pStorage, ItemSize, Capacity) | Initialize queue metadata and ring storage. |
| TaktOSQueueSend(pQue, pData, bBlocking, TimeoutTicks) | Copy one item into the queue or direct-handoff to a blocked receiver. |
| TaktOSQueueReceive(pQue, pData, bBlocking, TimeoutTicks) | Copy one item out of the queue or wait for a sender. |
| TaktOSQueueCount(pQue) | Return the current number of queued items. Use for observability, not flow control. |

# 7. Return Codes

| Code | Meaning |
| --- | --- |
| TAKTOS_OK | Operation completed. |
| TAKTOS_ERR_TIMEOUT | Finite wait expired before the operation completed. |
| TAKTOS_ERR_FULL | Queue/semaphore is full and non-blocking operation could not complete. |
| TAKTOS_ERR_EMPTY | Queue/semaphore is empty and non-blocking operation could not complete. |
| TAKTOS_ERR_BUSY | Mutex is already owned and non-blocking lock was requested. |
| TAKTOS_ERR_INTERRUPTED | Blocked wait was cancelled by Resume() or HandOff(); no token, lock, or item transferred. |
| TAKTOS_ERR_NOMEM | Object creation memory requirement was not met. |
| TAKTOS_ERR_INVALID | Invalid pointer, handle, state, caller context, ownership, or configuration. |
| TAKTOS_ERR_ALIGN | Pointer or size violated required alignment, usually queue storage/data alignment. |

# 8. Stack Protection

| Mechanism | Targets | How it works |
| --- | --- | --- |
| Guard word | All targets | TaktOSThreadCreate() writes 0xDEADBEEF after the TCB. Software check runs where hardware stack-limit protection is not active. |
| MPU guard region | MPU-capable Cortex-M ports | Runtime handler path can assign an MPU no-access region at the stack floor on context switch. |
| PSPLIM | Cortex-M33/M55 | Hardware process-stack lower bound is updated on context switch. |

void TaktOSStackOverflowHandler(hTaktOSThread_t hThread)
{
    (void)hThread;
    __BKPT(0);
}

NOTE  The application owns the vector table. Pass a writable HandlerBaseAddr only when the selected port supports runtime handler patching and the application has placed vectors in RAM.

# 9. POSIX and Standard Runtime Layers

## 9.1 POSIX PSE51 layer

The POSIX layer maps common PSE51 APIs onto native TaktOS primitives. It is useful for porting existing embedded code without changing the kernel scheduling model.

| Source | Functionality |
| --- | --- |
| src/posix/pthread.cpp | pthread_create, pthread_join, pthread_exit, pthread_self, attributes, scheduler priority mapping. |
| src/posix/sem.cpp | sem_init, sem_destroy, sem_post, sem_wait, sem_trywait, sem_timedwait. |
| src/posix/mqueue.cpp | mq_open, mq_close, mq_send, mq_receive, timed send/receive, mq_unlink. |
| src/posix/ptimer.cpp | timer_create, timer_delete, timer_settime, timer_gettime. |
| src/posix/clock.cpp | clock_gettime and clock_getres for monotonic/realtime clock IDs. |
| src/posix/errno.cpp | Per-thread errno location. |

#include "posix/pthread.h"
#include "posix/semaphore.h"
#include "posix/mqueue.h"
#include "posix/time.h"
#include "posix/ptimer.h"

## 9.2 ISO C threads and C++ runtime hooks

| File | Purpose |
| --- | --- |
| include/threads.h | ISO C <threads.h> front end for thrd_*, mtx_*, cnd_*, tss_* APIs. |
| src/std/threads.cpp | C threads implementation over the POSIX pthread substrate. |
| include/cxx/gthr-taktos.h | Draft libstdc++ gthr binding over pthread. |
| include/cxx/takt_cpp_runtime.h | Runtime hook declarations. |
| src/std/cpp_runtime.cpp | Weak runtime hooks for local-static guard locks and allocator entry points. |

NOTE  Place TaktOS/include before the toolchain system include path when an embedded target should resolve #include <threads.h> to TaktOS/include/threads.h.

- mtx_recursive currently returns thrd_error through the ISO C thread API.

- std::thread requires a libstdc++ build configured to use the TaktOS gthr binding.

- std::future, std::promise, and std::async require a bounded allocator policy before they should be considered production-ready.

# 10. Build-time Configuration Gates

| Symbol | Default | Effect |
| --- | --- | --- |
| TAKTOS_MPU_ENABLE | 0 | Enable MPU stack guard path where supported by the port. |
| TAKTOS_STACK_LIMIT | 0 | Enable PSPLIM stack-limit path on Cortex-M33/M55 where supported. |
| TAKT_DEBUG_CHECKS | unset | Enable extra internal checks during development builds. |
| TAKT_INLINE_OPTIMIZATION | unset | Force inline semaphore, mutex, and queue fast paths for highest throughput. Leave disabled for smaller code or coverage-oriented builds. |
| TAKT_TEST_ACCESS | unset | Expose test accessor functions. Unit-test only. |
| TAKTOS_MAX_HELD_PCP_MUTEXES | 4 | Maximum number of priority-ceiling mutexes one thread may hold simultaneously. |

# 11. Common Patterns

## 11.1 ISR to task notification

static TaktOSSem_t g_irq_sem;

void AppInit(void)
{
    TaktOSSemInit(&g_irq_sem, 0u, 1u);
}

void SENSOR_IRQHandler(void)
{
    TaktOSSemGive(&g_irq_sem, false);
}

static void SensorTask(void *arg)
{
    (void)arg;
    for (;;)
    {
        TaktOSSemTake(&g_irq_sem, true, TAKTOS_WAIT_FOREVER);
        ProcessSensor();
    }
}

## 11.2 Periodic task in milliseconds

static void PeriodicTask(void *arg)
{
    (void)arg;
    for (;;)
    {
        DoWork();
        TaktOSThreadSleep(TaktOSCurrentThread(), 10u);  /* 10 ms */
    }
}

## 11.3 Tick-domain wait

uint32_t tick_hz = TaktOSGetTickHz();
(void)tick_hz;
TaktOSThreadSleepTicks(TaktOSCurrentThread(), 1u);  /* next kernel tick */

## 11.4 Priority-ceiling shared resource

static TaktOSMutex_t g_bus_lock;

void BusInit(void)
{
    TaktOSMutexInitProtect(&g_bus_lock, TAKTOS_PRIORITY_HIGH);
}

void BusWrite(const void *data, uint32_t len)
{
    TaktOSMutexLock(&g_bus_lock, true, TAKTOS_WAIT_FOREVER);
    WriteBus(data, len);
    TaktOSMutexUnlock(&g_bus_lock);
}

# 12. KVB - Kernel Validation Benchmark

KVB is the on-target validation and benchmark harness in this baseline. It reports correctness state and optional performance metrics for the same test manifest across kernels and targets.

| Layer | Responsibility |
| --- | --- |
| KVB test modules | Scheduler, synchronization, IPC, timing, real-time responsiveness, and selected diagnostics. |
| KVB core harness | Registry, runner, assertions, metrics, logging, result model. |
| Kernel port | Map KVB primitives to TaktOS, FreeRTOS, ThreadX, or Zephyr APIs. |
| Platform port | Cycle/time source, UART/log output, IRQ probe, board metadata. |

## 12.1 Registered default tests

| Test ID | Group | Purpose |
| --- | --- | --- |
| SCHED_COOP_001 | SCHED | Cooperative yield throughput. |
| SYNC_SEM_FAST_001 | SYNC | Uncontended semaphore wait/post. |
| SYNC_MUTEX_FAST_001 | SYNC | Uncontended plain mutex lock/unlock. |
| SYNC_MUTEX_PCP_FAST_001 | SYNC | Uncontended priority-ceiling mutex lock/unlock. |
| SYNC_MUTEX_OWNERSHIP_001 | SYNC | Non-owner mutex unlock validation. |
| IPC_QUEUE_FAST_001 | IPC | Queue send/receive in one thread. |
| TIME_SLEEP_001 | TIME | Sleep accuracy across configured durations. |
| RT_SEM_WAKE_001 | RT | Semaphore wake-to-run latency. |
| RT_QUEUE_WAKE_001 | RT | Queue receiver wake-to-run latency. |
| RT_MUTEX_WAKE_001 | RT | Mutex waiter wake-to-run latency. |
| RT_TICK_JITTER_001 | RT | Sleep wake target jitter. |
| RT_IRQ_MASK_001 | RT | IRQ response and ISR-to-thread latency. |
| RT_TAKT_WAKE_TRACE_001 | RT | TaktOS-only diagnostic when KVB_ENABLE_TAKT_WAKE_TRACE_TEST=1. |

## 12.2 Result states

| State | Meaning |
| --- | --- |
| PASS | Expected behavior matched. |
| FAIL | Behavior violated the expected result. |
| UNSUPPORTED | Kernel or target does not provide the required primitive or behavior. |
| SKIPPED | Test was intentionally not run by configuration. |
| INVALID | Harness or platform configuration made the result unusable. |

[KVB] RESULT id=TIME_SLEEP_001 group=TIME state=PASS message="sleep duration valid"
[KVB] METRIC id=TIME_SLEEP_001 name=elapsed_us value=10023 unit=us
[KVB] SUMMARY pass=12 fail=0 unsupported=0 skipped=0 invalid=0 total=12

## 12.3 Configuration model

Use one test manifest and tune resources per target. Small-SRAM targets should reduce role-specific stack sizes, worker counts, queue depth, or RT iteration counts before disabling tests.

| KVB macro | Purpose |
| --- | --- |
| KVB_TICK_HZ | Kernel tick frequency used by tests, typically 1000. |
| KVB_CORE_CLOCK_HZ | Core or cycle-counter frequency used by timing conversion. |
| KVB_MEASUREMENT_MS / KVB_WARMUP_MS | Throughput window and warm-up time. |
| KVB_RUNNER_STACK_SIZE | Usable stack bytes for the KVB runner. |
| KVB_DEFAULT_STACK_SIZE | Default usable stack bytes for helper threads. |
| KVB_TEST_THREAD_STACK_SIZE | Shared helper-thread stack override. |
| KVB_SCHED_WORKER_STACK_SIZE | Scheduler worker usable stack override. |
| KVB_RT_THREAD_STACK_SIZE | Real-time test helper usable stack override. |
| KVB_SYNC_THREAD_STACK_SIZE | Synchronization test helper usable stack override. |
| KVB_WORKER_THREAD_COUNT | Cooperative scheduler worker count. |
| KVB_QUEUE_DEPTH / KVB_QUEUE_MESSAGE_SIZE | Queue resource shape used by IPC and RT queue tests. |
| KVB_ENABLE_RT_TESTS | Enable or disable RT responsiveness tests for targets with a usable timebase. |
| KVB_SLEEP_TEST_STRICT_MINIMUM | Keep 0 unless the sleep test aligns to a tick boundary before measuring. |

NOTE  Cortex-M0/M0+ targets have no DWT CYCCNT. Provide kvb_platform_time_us() using a board timer or report INVALID for timing-dependent tests rather than publishing meaningless latency numbers.

# 13. Scope Boundary

| Concern | Owner | Notes |
| --- | --- | --- |
| Scheduler, threads, semaphores, mutexes, queues | TaktOS | Core kernel responsibility. |
| Kernel tick and context switch exceptions | TaktOS port | SysTick/CLINT and PendSV/SVC/trap path as required by the architecture. |
| Application IRQ vectors and NVIC/PLIC setup | Application / HAL / BSP | TaktOS does not own application interrupt routing. |
| Clock tree and PLLs | Application / HAL / BSP | Configure SystemCoreClock or equivalent before TaktOSInit(). |
| Device drivers | IOsonata or another HAL | TaktOS does not configure I2C, SPI, UART, GPIO, BLE, USB, or storage drivers. |
| General memory management | Application or Partitura-class memory component | TaktOS kernel objects are static and caller-provided. |
| Application timers | HAL or application | Use TaktOSThreadSleep() for periodic tasks; general timers are outside the kernel. |

# 14. Migration Notes from Earlier Drafts

| Old usage | Rev 1.1 usage |
| --- | --- |
| TAKTOS_THREAD_STACK_SIZE(StackBytes) | TAKTOS_THREAD_MEM_SIZE(StackBytes). |
| TaktOSThreadSleep(thread, ticks) | TaktOSThreadSleep(thread, ms) or TaktOSThreadSleepTicks(thread, ticks). |
| Assuming all timeouts are milliseconds | Semaphore, mutex, and queue finite timeouts remain in ticks. |
| Plain mutex only | Plain mutex remains; priority-ceiling mutex is available through TaktOSMutexInitProtect(). |
| Documenting RISC-V as supported | Treat RISCV/ as a placeholder/design area for this baseline, not a v1.x production port. |