TaktOS

RTOS Performance Benchmark Report

ARM Cortex-M4 / nRF52832  ·  Rev 2.0  ·  April 2026

# 1. Executive Summary

TaktOS is a bare-metal deterministic RTOS for ARM Cortex-M and RISC-V RV32, targeting IEC 61508 SIL 2 and ISO 26262 ASIL D certification. This report presents directly measured Thread-Metric benchmark results comparing TaktOS, FreeRTOS, and Eclipse ThreadX on identical hardware with identical build tooling. ThreadX was measured with source code access, not a prebuilt binary.

Platform: I-SYST BLYST Nano (nRF52832)  ·  64 MHz Cortex-M4F  ·  arm-none-eabi-gcc 15.2.1  ·  -Os  ·  1 kHz tick

All three RTOSes built with identical flags and linker script on the same board. No cross-board normalization. No estimates.

Key results at a glance:

- 5.18× faster than FreeRTOS on TM7 synchronization (semaphore throughput)

- 4.01× faster than FreeRTOS on TM6 message processing (queue throughput)

- 2.15× faster than FreeRTOS on TM3 preemptive scheduling

- 1.93× faster than FreeRTOS on TM2 cooperative scheduling (FreeRTOS produced correctness errors)

- 1.39× faster than source-tuned ThreadX on TM7 synchronization

- 3.05× geometric mean advantage vs FreeRTOS  ·  1.35× vs ThreadX (TM2–TM7)

- 38% smaller binary than FreeRTOS on the TM7 synchronization build (5,618 vs 9,096 bytes .text)

# 2. Results Summary

Steady-state 30-second iteration counts. Higher is better. Values shown are the stable window count after thermal settling.

| Test | TaktOS | FreeRTOS | ThreadX ² | T / FR | T / TX |
| --- | --- | --- | --- | --- | --- |
| TM1 Basic Processing (calibration) | 124,658 | 124,679 | 124,702 | — | — |
| TM2 Cooperative Scheduling | 16,349,937 | 8,473,241 ⚠ | 10,660,764 | 1.93× | 1.53× |
| TM3 Preemptive Scheduling | 5,095,925 | 2,366,640 | 4,396,380 | 2.15× | 1.16× |
| TM6 Message Processing | 9,034,249 | 2,251,869 | 6,749,360 | 4.01× | 1.34× |
| TM7 Synchronization | 21,289,905 | 4,112,298 | 15,334,262 | 5.18× | 1.39× |
| Geometric mean (TM2–TM7) |  |  |  | 3.05× | 1.35× |

⚠  FreeRTOS TM2: produced repeated 'counters more than 1 different from average' determinism check errors. TaktOS and ThreadX: zero determinism errors across all tests and all measurement windows.

²  ThreadX tested with source code. Not a prebuilt binary.

TM1 note: all three RTOSes score within 0.04% (124,658–124,702). No context switches occur during TM1 — this test calibrates the measurement environment, not RTOS performance.

TM4 and TM5 are not run: TM4 requires kernel-owned hardware IRQs; TM5 requires dynamic memory allocation. Both conflict with TaktOS architectural constraints.

## Binary Size

Synchronization test (TM7) build — .text section only.

| RTOS | .text bytes (TM7) | vs TaktOS |
| --- | --- | --- |
| TaktOS | 5,618 | — |
| ThreadX ² | 6,205 | +10% |
| FreeRTOS | 9,096 | +62% |

# 3. Per-Test Results

## 3.1  TM2 — Cooperative Scheduling

Five threads yielding cooperatively via tm_thread_relinquish(). Measures cooperative yield throughput and round-trip scheduling latency.

| RTOS | Score | vs TaktOS | Notes |
| --- | --- | --- | --- |
| TaktOS | 16,349,937 | baseline | CLZ bitmap + inlined TaktOSCtxSwitch(); tight PendSV body |
| ThreadX ² | 10,660,764 | 0.65× | Precomputed execute_ptr — same architecture as TaktOS |
| FreeRTOS | 8,473,241 ⚠ | 0.52× | BASEPRI critical sections + non-precomputed scheduler |

⚠  FreeRTOS TM2 produced repeated determinism check errors on this board.

## 3.2  TM3 — Preemptive Scheduling

Five threads at different priorities. Higher-priority thread preempts lower-priority on resume. Measures preemption latency and context switch throughput.

| RTOS | Score | vs TaktOS | Notes |
| --- | --- | --- | --- |
| TaktOS | 5,095,925 | baseline | PendSV preemption + precomputed next-thread pointer |
| ThreadX ² | 4,396,380 | 0.86× | execute_ptr design — competitive on preemptive test |
| FreeRTOS | 2,366,640 | 0.46× | BASEPRI + list walk on every preemption |

## 3.3  TM6 — Message Processing

Two threads exchanging fixed-size messages via a queue. Measures queue send/receive throughput including blocking/unblocking overhead.

| RTOS | Score | vs TaktOS | Notes |
| --- | --- | --- | --- |
| TaktOS | 9,034,249 | baseline | Direct-pointer queue; TaktQueueFastCopy word-unrolled copy; inlined fast path |
| ThreadX ² | 6,749,360 | 0.75× | Mature queue implementation — competitive |
| FreeRTOS | 2,251,869 | 0.25× | BASEPRI + vListInsertEnd overhead on every send/receive |

Long-duration determinism (TM6): TaktOS TM6 was run continuously for 71,940 seconds (2,131 consecutive 30-second measurement windows, approximately 20 hours). Every window produced either 9,038,536 or 9,038,537. The 1-count spread is tick-boundary sampling jitter, not drift. No degradation, no anomalies across the entire run. Bit-exact determinism confirmed under sustained load on real hardware — directly relevant to IEC 61508 SIL 2 temporal behavior requirements.

## 3.4  TM7 — Synchronization Processing

One thread performing semaphore get/put in a tight loop. Measures uncontended semaphore throughput — the most sensitive test for critical-section and fast-path overhead.

| RTOS | Score | vs TaktOS | Notes |
| --- | --- | --- | --- |
| TaktOS | 21,289,905 | baseline | TaktOSSemGive/Take always_inline; PRIMASK 2-instruction critical section |
| ThreadX ² | 15,334,262 | 0.72× | Tight semaphore implementation — strong result |
| FreeRTOS | 4,112,298 | 0.19× | BASEPRI DSB/ISB flush penalty on every semaphore operation |

# 4. Why TaktOS Is Faster

## 4.1  vs FreeRTOS (3.05× geometric mean)

- CLZ priority bitmap — __builtin_clz on a 32-bit bitmap gives O(1) highest-priority lookup. Updated at event time; context switch handler reads one precomputed pointer.

- PRIMASK critical sections — MRS + CPSID / MSR: 2 instructions, no pipeline flush. FreeRTOS BASEPRI requires DSB + ISB after every write (~8 cy penalty per boundary on Cortex-M4).

- Inlined semaphore fast path — TaktOSSemGive and TaktOSSemTake are always_inline static functions. Uncontended path executes with zero function-call overhead — dominant gain on TM7.

- Direct-pointer queue — writePtr/readPtr are direct buffer addresses. Slot address is one load. TaktQueueFastCopy uses switch-unrolled word assignments for 1–8 word messages.

- Prefetch-buffer fit — PendSV_Handler body (36 bytes, .map confirmed) fits within the nRF52832's 128-byte I-code prefetch window.

## 4.2  vs ThreadX (1.35× geometric mean, source-tuned)

ThreadX uses a precomputed execute_ptr updated on every ready/block operation — the same architecture as TaktOS. The gap is narrower than vs FreeRTOS. TaktOS leads most strongly on TM2 cooperative (1.53×) and TM7 synchronization (1.39×), where the tighter semaphore fast path and PRIMASK critical sections provide the clearest advantage.

# 5. TaktOS Product Overview

TaktOS is a commercial bare-metal RTOS developed by I-SYST inc., engineered for deterministic performance and functional safety certification on ARM Cortex-M and RISC-V RV32 targets.

- ~590 LOC portable C++23 scheduler core — MC/DC certification burden in weeks, not months

- ~1,454 LOC total auditable boundary (ARM + portable C++23 + Thumb-2 assembly)

- Static allocation only — zero heap in hot paths; all objects user-supplied

- POSIX PSE51 layer (pthread, sem_t, mqueue, timer) outside cert boundary

- IOsonata DevIntrf_t driver model — identical on ARM and RISC-V

- IEC 61508 SIL 2 target (SEooC) with path to ISO 26262 ASIL D

- MIT licensed

## Supported targets

- ARM: Cortex-M0/M0+, M4/M4F, M7, M33, M55

- RISC-V: RV32IMAC machine mode (GD32VF103, FE310) — Zbb recommended

- RISC-V: ESP32-C3 (RV32IMC) and ESP32-C6 (RV32IMAFC + Zbb)

## Certification cost estimate

Dual-arch SIL 2 (ARM + RISC-V): $600K–$1.0M, 9–12 months. ASIL D uplift: +$300K–$500K, +6 months. Certifying company owns the IP. Small cert boundary is the core cost advantage vs FreeRTOS or Zephyr.

# 6. Test Methodology

## 6.1  Hardware

I-SYST BLYST Nano (IMM-NRF52832-NANO) System-on-Module. Nordic Semiconductor nRF52832 SoC — ARM Cortex-M4F @ 64 MHz, 512 KB Flash, 64 KB RAM, hardware FPU. No on-chip instruction cache. Module: 10 × 7 × 1.6 mm. Designed and manufactured by I-SYST inc., Brossard, Canada.

## 6.2  Build configuration

- Compiler: xPack GNU Arm Embedded GCC 15.2.1 (arm-none-eabi)

- Optimization: -Os (size-optimized, consistent across all three RTOSes)

- Tick rate: 1000 Hz SysTick

- Stack per thread: 1024 bytes

- ThreadX: tested with source code — not a prebuilt binary

## 6.3  Benchmark suite

eclipse-threadx/threadx Thread-Metric benchmark suite (utility/benchmarks/thread_metric/). MIT license. Source unmodified. Tests TM1–TM3 and TM6–TM7 used.

TM4 and TM5 are not run for TaktOS: TM4 requires the harness to own a hardware timer IRQ (TaktOS does not own application IRQs by design). TM5 measures dynamic allocation (TaktOS has no heap by design).

## 6.4  Measurement validity

Each score is the stable 30-second window value after thermal settling. TM6 long-duration determinism: TaktOS TM6 (message processing) was run continuously for 71,940 seconds — 2,131 consecutive 30-second measurement windows, approximately 20 hours. Every window produced either 9,038,536 or 9,038,537. The 1-count spread is tick-boundary sampling jitter, not drift. No degradation, no anomalies across the entire run. This confirms bit-exact determinism under sustained load on real hardware. TM3 was run for 1,590 seconds (53 windows), showing a 3-count spread attributable to thermal settling in the first 1,050 seconds, then locking to a stable value for the remaining 540 seconds.

# 7. References

- eclipse-threadx/threadx — Thread-Metric benchmark suite. MIT license.

- ARM Cortex-M4 Technical Reference Manual (r0p1) — exception entry/exit cycle counts.

- Beningo Embedded Group — RTOS Performance Report 2024. STM32L475 @ 80 MHz, GCC 12.3. Referenced for FreeRTOS determinism failure cross-reference only.

- Mazzi et al. (2021) — Benchmarking and Comparison of Two Open-source RTOSs. STM32F429 DWT measurements. Referenced for RTX5 context switch figure (~140 cycles).

- xPack GNU Arm Embedded GCC 15.2.1 — https://github.com/xpack-dev-tools/arm-none-eabi-gcc-xpack

All TaktOS, FreeRTOS, and ThreadX measurements produced directly on the I-SYST BLYST Nano at I-SYST inc. labs, April 2026. Eclipse project files available at github.com/I-SYST/TaktOS-ThreadMetric.